Pfsense Configuration & Subnetting

Part 1: Initialization of subnets

After spending a considerable amount of time fighting with Neutron/Openstack architecture, I have elected to start working with it. The intended architecture is to encapsulate the entrypoint to the VSOC in an (ext <-> router <-> transit net/29 <-> pfsense wan NIC). The purpose of this is to make use of all of the architecture that is already provided by openstack to manage our project. This helps openstack understand the routing through our project and allows us more wiggle room with design decisions. For example, we can NAT through pfsense or through the transit net neutron router. Additionally, I used an ext attached floating-IP to allow xrdp into lan net to make using the web configurator(GUI) possible.

Procedure Openstack CLI tool is downloaded to  and used on local machine

openstack network create transit-net // create buffer network

openstack subnet create --network transit-net --subnet-range 10.250.2.0/29 --gateway 10.250.2.1 transit-subnet       // create subnet and attach it to the network

openstack router create ex-router // create router to connect ext and transit. Will take gatewayIP

openstack router add subnet ex-router transit-subnet

openstack router set --external-gateway ext_net ex-router

Through Openstack GUI, Create Ubuntu Instance rdp-client. Attached to transit.

DNS wasn’t working so make it work:

openstack subnet set transit-subnet \\

--dns-nameserver 10.2.3.245

openstack subnet set lantest-subnet \\

--dns-nameserver 10.2.3.245

I did a bunch of weird stuff to fix hostname resolution and dns resolution on the rdp-client

Now DNS resolution and Egress traffic has been tested and works. It is time to set up the internal network that will be the next downstream.

In openstack GUI, create a network called lantest-net, subnet called lantest-subnet, subnet range = 105.45.5.0/24. Gatewayip = 105.45.5.1.

Lastly We create one final host vm in lantest-subnet for testing our future pfsense configuration.

Using the GUI, create a simple ubuntu instance attached to lantest-subnet called ‘interna;’, size m1-medium in case we want to use it for different purposes in the future. The IP ends up being 105.45.5.72

Now it is time to create Pfsense

Part 2: Pfsense creation

In this section our goal is to create a pfsense instance with properly configured ports. We will test our functionality by sending/tracing imp traffic egress towards dns names like google.com, nodes attached to ext like ext_testinstance, and laterally to rdp-client (within the same subnet). We will also make sure that we can ping our internal member ‘internal’ to ensure both interfaces are properly configured.

Procedure

Create a Pfsense VM. M1.medium volume, default security group(allow all)

Openstack attaches fixed ips to the our interfaces by mac address before we configure anything inside of pfsense. We need to make sure that we are associating the correct interface with the correct subnet by corroborating our assignments with the mac addresses attached in openstack.

To get the openstack ip assignments of pfsense interfaces:

openstack port list --server newpfsense  // returns table of portID -  ip - mac - name

//  name the ports

openstack port set --name pfw 2f2c6f3b-0297-473f-8cec-a117b3ac291d

openstack port set --name pfl 2320660f-c232-4bd8-9c41-b1486046dfab

//Additionally, pfsense will be the upstream gateway for lantest-subnet, so we need lanIP to = gatewayIP. Make the modification in openstack.

Openstack port set –no-fixed-ip pfl  // remove current ip

Openstack port set –fixed-ip subnet=lantest-subnet.ip-address=105.45.5.1 pfl //  set new IP address to default gatewayIP

Now to crossreference with pfsense inside pfconsole:

Option 8 enter shell:

Ifconfig vtnet0  // look at MAC Address. This will correspond to LAN or WAN ports in OS

Assign interfaces by matching mac of vtnet0 with which pool it’s ip is in. Addresses vtnet1->lan, vtnet0->wan

Assign IP addresses inside of pfsense:

Non-DHCP allocation.

LanIP->lan-subnet(105.45.5.1/24). HostIP matches openstack, set no upstream, turn off ipv6, enable dhcp server for LAN: allowed range(105.45.5.4 - 105.45.5.254). Do not revert to HTTP for web configurator (web GUI)

Web GUI address: ‘https://105.45.5.1’

Non-DHCP allocation. (even though you could)

WANIP->ext-subnet(10.250.2.5/29). Match the Openstack IP.

Set upstream->ext_subnet gateway ip(10.250.2.1). This is the Neutron router. Set this as default gateway.Turn off ipv6. Do not enable DHCP. Do not Revert to HTTP

Change user admin passwd to ‘123456789’ in UI menu

After significant time spent tooling around, Pings and Traceroutes going upstream, downstream, and even laterally should work both ways.

GUI Access

The Goal of this section is to get rdp traffic from inside of the lan network out to ext through the medium of a floating IP. Recall that rdp-client is placed laterally inside of transit net next to pfsense. The temporary solution of this step is to create a second NIC from rdp-client into lantest-net so that we can configure pfsense before it has bastion functionality. Additionally, we will want to turn off NAT unless we decide to disable neutron snat in ext-router.

Procedure

Attach ext-pooled floating IP to rdp-client (possible due to neutron server). This is done in the OS GUI.

Ssh into rdp-client using .pem file

Add pub key to authorized_keys

Sudo passwd ubuntu // change to 123456789

// We will be installing terminals, browsers, and rdp functionality

Kleinjw2_testubuntu side

- sudo apt install -y xorg xfce4 xfce4-goodies firefox xrdp  //(takes some time with 1 vcpu)

- sudo apt update

- sudo apt full-upgrade -y

- sudo apt autoremove --purge -y

- sudo reboot

- systemctl status xrdp 		//(look for enabled, running) else enable then reboot

- sudo adduser xrdp ssl-cert

- sudo systemctl restart xrdp

- echo "startxfce4" > ~/.xsession

- chmod 644 ~/.xsession

Local Desktop side(arch btw)

- sudo pacman -S freerdp freerdp2

NIC ADDITION

openstack port create --network lantest-net --fixed-ip subnet=lantest-subnet,ip-address=105.45.5.5 rdpnic  // create lan port

openstack server add port rdp-client rdpnic  // attach to client

//verify

openstack port list --server rdp-client

Client Side:

Update netplan

sudo vim /etc/netplan/50-cloud-init.yaml

Update file to look like: (ens4 interface is what we've added)

network:

version: 2

ethernets:

ens3:

match:

macaddress: "fa:16:3e:6c:31:68"

dhcp4: true

set-name: "ens3"

ens4:

match:

macaddress: "fa:16:3e:fd:1d:12"

addresses: [105.45.5.5/24]   # your LAN_IP/CIDR

routes: []                      # no default route

set-name: "ens4"

sudo netplan apply

// we have added port handling so that it does not interfere with primary port dhcp, and is generally ineffectual to primary processes

Run locally:

xfreerdp /v:172.17.187.15  /u:ubuntu /cert:ignore

Passwd: 123456789

Open Firefox->https://https://105.45.5.1->pfsense login

User:admin

Passwd:123456789

It Works

PFsense Configurations

// Stop blocking private networks

Interfaces/Wan: Disable/untick Block private networks and loopback addresses

// Pass Pings

/Firewall/Rules/Wan: Allow ICMP, subtype: Echo Request, Any source/destination.

#Keep NAT on. We need to NAT our internal network in order for return traffic to reach lan. Once #we are able to use a vpn, and can delete the rdp-client, we can disable snat on ex-router

Openstack Static Routing Configuration

Openstack actually handles a lot of its own routing pretty well. The only thing we need to add is for routing to our lantest subnet(isolated from neutron) to go through transit-net. This will allow ingress traffic to find the correct path to our isolated network.

//  10.250.2.5 is the ip of our pfsense wan interface that we are sending traffic to

openstack router add route --route destination=105.45.5.0/24,gateway=10.250.2.5 ex-router

An additional note is that there is no direct way to route our local machine’s traffic to target subnets through the proper ext-gateway because it is not on link. This means that we will need to ssh proxyjump from ext -> pfsense -> lantest. Pfsense allows agent forwarding.

We also configure ext_testinstance to route through ex-router(on link)

ip route add 10.250.2.0/29 via 172.17.184.110 dev ens3

SSH Bastion Implementation

On PFSENSE GUI

Keep NAT Enabled. In the future we should change the ssh TCP port.

System->Advanced->Enable Secure Shell and Agent Forwarding

Firewall/Rules/edit-> Pass WAN TCP from any source/destination on Ssh Port 22, Allow all ICMP everywhere.

Add public key to admin user:     SystemUser ManagerUsersEdit: paste into authorized_keys.  // on gui

We can create a non-admin user for our proxy for more organization.

Ssh into pfsense to test.

In .ssh/config: add host for internal, pfsense. Proxyjump into internal through pfsense with pem file initially, add your user key to authorized keys then update local .config

#.ssh/config

Host pfsense

HostName 172.17.186.187

User admin

IdentityFile ~/.ssh/id_ed25519

IdentitiesOnly yes

Host open_int

HostName 105.45.5.72

User ubuntu

ProxyJump pfsense

IdentityFile ~/.ssh/id_ed25519

IdentitiesOnly yes

Ssh should work for internal ports now.

Wireguard Implementation